BEFS RAPID APPRAISAL EXERCISE

Agricultural Residues Component

Introduction.

The Republic of the Philippines is an archipelago of over 7,000 islands in the Pacific Ocean. With a population of around 94 million inhabitants, the country imports about 45% of its energy needs. In this context, the Philippine government enacted an ambitious renewable energy act (R.A. No. 9513) in July 2008. Under this regulatory framework, the “National Renewable Energy Program (NREP)” was developed. The program’s objective is to increase energy security through diversification of energy sources, increase electricity coverage and improve the use of clean energy. Decentralised electricity production is also supported through this Program.

The Island of Basilan, located in the Autonomous Region of Mindanao, currently uses a diesel generator to supply electricity to its inhabitants. The supply is intermittent due to problems in delivery of diesel from the other islands.

The island has a stable production of rice, which is processed locally. The rice husk, generated in the mills could be used for electricity generation, and thus ensure sustainable supply of electricity on the island.

In order to determine the feasibility of local electricity production using rice husk, the first step is to estimate the availability and sustainability of supply of the feedstock (rice husk) and then to analyse techno-economic viability of such a project.

In the Part 1 of this exercise we will assess the availability of rice husk for energy production in the Island of Basilan, taking into account other current uses.

After that, In the Part 2 we will evaluate the feasibility of generation and distribution of electricity using combustion technology.

Part 1 Agricultural residues availability

Like the majority of countries in Southeast Asia, rice is the staple food in the Filipino diet. Practically all of the rice that is produced is intended for local consumption.

Initial Instructions.

- Go to the Excel file “Agricultural Residues tool 1.0.xlsm” (remember to enable Macros and ActiveX if asked).

- Scroll down and click on the “NEXT >> Agricultural Residues Component”, using the navigation button.

- Scroll down and go to the “Crop Residues Tool”, using the navigation button.

- Select the following crop for analysis: Crop 2: rice.

- Information about the Residue type to be analysed and the Location of residue generation are mentioned in the introduction of this handout.

- Based on the descriptions given below for rice crops and residues (Parts 1a, 1b and 1c), enter the data needed by the tool to run the analysis: crop-residue type, location of residue generation, the current situation on crop production, residue to crop ratio, residues left in the field, residues burnt in the field, and the current use of crop residues.

Note: In the column for Crop 1 you will find data from the example used during the presentation (rice straw). For the exercise use the column for Crop 2. If the data needed is not in the text below, then leave the cell blank.

Use the following screenshot as a guide when entering the information:

Part 1a	. Crops and crop production

Rice is harvested once per year in Philippines. Data from the Philippines Statistics Authority on rice production in the Island of Basilan indicates that for the year 2014 rice had an average crop yield of 2.46 tonnes/ha and an average production of 4.800 tonnes/year.

Part 1b. Generation of residues

Once rice has been harvested in the field is transported to rice mills. Milling is a crucial step in production of rice since it removes the husk and produces an edible rice kernel sufficiently free of impurities.

As it can be noted, rice husk residues are produced in the processing plant. The residue to crop ratio of rice husks is 0.20 in the Philippines.

Part 1c. Current uses of crop residues

As to ensure that the use of rice husk doesn’t impose undesired changes in the local communities, the currently used rice husk should not be considered as available for energy production. Therefore, the current uses are recorded, and the amount used subtracted from the total amount of husk generated in the mills.

Current uses of rice husk residues in the Island of Basilan are:

- Animal feed and bedding for livestock: 16%

- Fuel: 0.8%

- 9.21% is sold in the market.

Enter these values in the part 1c and answer the questions below.

Agricultural residues results.

How many tonnes per year of rice husk residues are potentially available for bioenergy generation in the Island of Basilan?  ___________

What is the residue yield? ________

To answer this question, go to the “Results of the Agricultural Residues” section using the navigation button situated at the bottom of the sheet and review the results.

Finally, scroll down to the section “Define the allocation of Crop Residues for bioenergy production” and assign 100% of the available rice husk residues to the alternative of Rural Electrification with a Combustion technology as shown in the following screenshot: