BEFS RAPID APPRAISAL EXERCISE

Combustion Component

Introduction

As mentioned before, the Island of Basilan currently uses diesel generators for electricity generation because of its location. However, provision of electricity is limited to certain times during the day and in many cases it has to be discontinued due to lack of fuel.

Lack of diesel is caused by difficulties in the supply of diesel and the higher price of this fuel due to transportation cost from neighbouring islands.

In this part of the exercise we will consider whether the technological alternative of combustion of available rice husk residues for electricity generation is sustainable and economically feasible in the context of the Island of Basilan. For this, use the BEFS RA Combustion Tool to analyse different production scenarios.

Part 2 Combustion for rural electrification

Initial Instructions.

- Open the Excel file “Combustion_tool_1.0.xlsm” (remember to enable Macros and ActiveX).

- From the START page using the navigation controls click on “NEXT >> Energy Demand”

- Follow Parts 2a to 2g to input all the necessary information required by the tool to complete the analysis.

Note: In the column for Crop 1 you will find data from the example used during the presentation (rice straw). For the exercise use the column for Crop 2. If the data needed is not in the text below, then leave the cell blank.

Part 2a: “Energy Demand” Section.

To start the analysis the electricity consumption per household must be entered

The following screenshot shows the electricity consumption profile of an average household in the Island of Basilan

When finished, click “NEXT >> Data Entry”.

Part 2b: “DATA ENTRY FOR COMBUSTION” Section.

Go to the sub-section “Feedstock Availability and Cost” and complete the information indicated in the following screenshot:

Information on Part 2b should be completed using the results from the Crop Residues analysis performed in the part 1 of the exercise. A summary of the results is shown in the following screenshot:

Part 2c: “DATA ENTRY FOR COMBUSTION” Section.

For this exercise, we assume that the rice mill itself will be responsible for generating electricity. Therefore, the cost for purchasing feedstock (rice husks) at a market price and collection costs are zero. The cost of feedstock storage is estimated to be 3 USD/t.

Part 2d: “ELECTRICITY PRICE DEFINITION”

Scroll down until you reach the section “Electricity price definition”.  For this project, it is initially suggested to compare electricity produced against the price paid in the regions connected to the central grid: 0.1 USD/kWh.

To do this, go to the section “Electricity Price Definition” and select “Method 2” and enter the value as seen in the following screenshot:

When finished, click “NEXT >> Summary of Results by Feedstock”.

Group Discussion.

Analyse and discuss the results obtained in the “Summary of Results by Feedstock” within your working group. Answer the following:

- How many households could be potentially supplied?  ___________

- Is this alternative a cost effective option? _______________

Part 2e: Electricity Price Comparison

We have seen how the projected price paid by consumers is the same price paid in the central islands where electricity is produced. The economic performance of this project is disastrous. However, we know that this island currently uses power plants that run on diesel transported from another island 100 km north.

Click on the button “BACK >> Data Entry” and return to the section “Electricity Price Definition”. In order to compare our results with the actual price paid for electricity on this island we need to it with a comparable alternative for electricity generation, in this case, diesel generation.

For this, go to the section “Electricity Price Definition”  Select “Method 1”. Click on the “Electricity Price Calculator” button and enter the following information:

When finished, click “Calculate”. Then review in what way the results have changed by clicking “NEXT >> Summary of Results by Feedstock”.

Group Discussion.

- If we compare the combustion technology with generation using diesel, is combustion a cost effective option? ____________

Part 2f: Electricity Demand Comparison

A common phenomenon that occurs in rural communities when they access to a stable electricity service is that electricity demand per household increases since people acquire new electrical appliances and gradually increases the use of currently owned appliances. This should be taken into account when designing an electricity provision system.

The following screenshot shows the electricity consumption profile of an average household in the Island of Basilan considering increased electricity consumption and new appliances:

Click on the button “BACK >> Data Entry” and then on the button “NEXT >> Energy Demand”. Enter the new energy demand. Review in what way the results have changed by clicking on “NEXT >> Data Entry” and then “NEXT >> Summary of Results by Feedstock”.

Group Discussion.

- How many households could now be potentially supplied?  ___________

Part 2g: Summary of results comparative

So far we have reviewed the results obtained for rice husk residues. However, the combustion tool has been already filled with the data for rice straw residues.

Click the button “NEXT >> Summary of Results Comparative” to compare the results from these two types of residues.

Group Discussion.

- Which of the two feedstock shows the most promising results for the Island of Basilan? For this, consider:

  - The comparison of production costs (US$/kWh)

  - Comparison of total number of households that could be supplied

  - Financial results for both alternatives